Date: 11/11/2022

Information: Lampros Petropoulos

Tel: 2310-365815, 6973494864

e-mail: lpetropoulo@herrco.gr

TECHNICAL SPECIFICATIONS of a portable-robotic Material Recovery Facility

- General description

The project“RECLAIM: AI-powered Robotic Material Recovery in a Box”, funded under HORIZON-CL4-2021-DIGITAL-EMERGING-01-09 (EU GA: 101070524) concerns the supply, manufacture and assembly of a portable, robotic Material Recovery Facility (prMRF) to be used for research and innovation purposes. Sorting process will be done by six (6) cartesian robots installed in a container box. The prMRF will be fed with post-consumer waste (packages) of various material types (plastics: PET; HDPE; PP; PS and film, tetrapack, aluminum and iron).

The equipment that will be required in order to implement the RECLAIM prMRF is a) Container box to host all installations, b) Inclined feeding conveyor belt c) Vibrating material distributor d) Forward conveyor belt that conveys materials for sorting e) Magnet f) Return conveyor belt g) Inclined return conveyor belt h) others.

- Container box

| Description of equipment | Container |
| --- | --- |
| Type | 40'High Cube Pallet Wide Container |
| Internal length (mm) | 12.100 mm |
| Internal width (mm) | 2.422 mm |
| Internal height (mm) | 2.695 mm or taller |
| Doors | Οn both short container sides |
| Windows (openings) | 3 aluminum windows on each side of the long container sides at locations (from container start): 1.5m [window dimension: 600mm x 2000mm (HxW)], 5.0m [window dimension: 600mm x 3000mm (HxW)] and 9.5m [window dimension: 600mm x 2000mm (HxW)] |
| Discharge holes | 13 openings and placement of sorting chutes for the collection of selected material. |
| Support equipment for the cartesian robots. | Placement of 6 horizontal hollow beams parallel to each other and vertically to the long side of the container. The hollow beams will be installed at 5.3m, 6.2m, 9.5m, 10m, 10.8m, 11.4m across the long container side and will be at 35-50 cm above the forward conveyor belt. |

| Description of equipment | Container |
| --- | --- |
| Painting | Painted white (outside, inside) |
| External coating | White panel 3 cm thick and RAL 9002 |
| Electrical Installation | Electrical installation with electrical panel and three-phase sockets, LED lights, safety buttons. All wiring will be in plastic tubes or grids. 3 electrical panels (1400 mm x 1000mm x 400mm). |
| Container support | 4 Hydraulic lifting bases and the corresponding support base for permanent installation |

- Inclined feeding conveyor belt

Construction of a portable belt that will feed the vibrating distributor. It will consist of a horizontal section and an inclined section. For this specific conveyor belt, the following should be clearly described:

- Total length

- Useable width

- Max Width (mm)

- neck

- Inclination

- Belt

- Motor

- Underside covering

- Steel construction

- Drive unit

- Slave unit

- Vibrating material distributor

Vibrating distribution system or equivalent distribution system, which removes dirt and small unwanted pieces from recyclables and distributes materials across the whole width of the forward conveyor belt. Full description/specification needed.

- Forward conveyor belt (inside container)

| Description of equipment | Forward conveyor belt |
| --- | --- |
| Total length (m) | approx. 10-11 m |
| Useable width (mm) | 1100 mm |
| Max Width (mm) | 1200 mm |
| Height (upper belt surface-floor) (mm) | 1100 mm |
| Inclination | 0o |
| Conveying velocity (m/s) | in the range 0.1-0.5 m/sec |
| Inverter | Yes |
| Belt | EP 250/2 with coverage 3+0 mm oil and grease resistant. including vulcanized attached devices |
| Motor (Drive Unit) | Attachable gear motor Voltage 230/400 V 50Hz |
| Underside covering | Closed / full covered frame |
| Steel construction | Rigid steel construction, U type profiles of 2.5 mm thick sheet |
| Drive unit |  |
| Drum diameter | approx 200 mm |
| Rubber coating diameter | diamond 10mm |
| Slave unit |  |
| Drum diameter | approx 200 mm |
| Rubber coating diameter | no |
| Βelt tension (tightening type) | Spindle tension system (pin) |
| Rollers (up side & under side) |  |
| under/up side rollers (returning) distance | 1500 mm |
| Τype of rollers | rigid axle with ball - bearings disk (rubber coated). Referances necessary |
| Bearing | Bearings should be available in the domestic market |
| Type | solid shaft - rubberized running discs |
| Emergency stop | Yes |

- Magnet - Ferrous separation

The magnet should be positioned between the 2rd and 3th horizontal hollow beams. Max width 1 m

- Return conveyor belt (inside container)

| Description of equipment | Return conveyor belt |
| --- | --- |
| Total length (m) | approx. 8 m |
| Useable width (mm) | 700 mm |
| Max Width (mm) | 750 mm |
| Height (down side conveyor -floor) (m)) | 0.4 m |
| Inclination | 0o |
| Conveying velocity (m/s) | in the range 0.1-0.4 m/sec |
| Inverter | Yes |
| Belt | EP 200/2 with coverage 2+0 mm oil and grease resistant. including vulcanized attached devices |
| Motor (Drive Unit) | Attachable gear motor Voltage 230/400 V 50Hz |
| Underside covering | Closed / full covered frame |
| Steel construction | Rigid steel construction, U type profiles of 2.5 mm thick sheet |
| Drive unit |  |
| Drum diameter | approx 150 mm |
| Rubber coating diameter | diamond 10mm |
| Slave unit |  |
| Drum diameter | Approx. 150 mm |
| Rubber coating diameter | no |
| Βelt tension (tightening type) | Spindle tension system (pin) |
| Rollers (up side & under side) |  |
| under/up side rollers (returning) distance | 1500 mm |
| Τype of rollers | rigid axle with ball - bearings disk (rubber coated). Referances necessary |
| Bearing | Bearings should be available in the domestic market |
| Type | solid shaft - rubberized running discs |
| Emergency stop | Yes |

- Inclined return conveyor belt (inside container)

| Description of equipment | Inclined return conveyor belt |
| --- | --- |
| Total length (m) | approx. 4m -5 m |
| Useable width (mm) | 500 mm |
| Max Witdh (mm) | 550 mm |
| Height (down side conveyor -floor) (m)) | 0 m |
| Inclination | ~ 40o |
| Conveying velocity (m/s) | approx 0.1-0.4 m/sec |
| Inverter | Yes |
| Belt | EP 200/2 with coverage 2+0 mm oil and grease resistant. including vulcanized attached devices |
| Belt cleats | Yes |
| Motor (Drive Unit) | Attachable gear motor Voltage 230/400 V 50Hz |
| Underside covering | Closed / full covered frame |
| Steel construction | Rigid steel construction, U type profiles of 2.5 mm thick sheet |
| Drive unit |  |
| Drum diameter | approx 150 mm |
| Rubber coating diameter | diamond 10mm |
| Slave unit |  |
| Drum diameter | Approx. 150 mm |
| Rubber coating diameter | no |
| Βelt tension (tightening type) | Spindle tension system (pin) |
| Rollers (up side & under side) |  |
| under/up side rollers (returning) distance | 1500 mm |
| Τype of rollers | rigid axle with ball - bearings disk (rubber coated). Referances necessary |
| Bearing | Bearings should be available in the domestic market |
| Type | solid shaft - rubberized running discs |
| Emergency stop | Yes |

- Other equipment (inside container)

- Discharge hoppers: 13 hoppers with an adjustable slide for discharge (a) to the bottom of the container and (b) to the return belt.

- Discharge slide: at the end of the forward conveyor belt adjustable for discharge (a) to outside of the container and (b) to the return belt.

- 2 Slides: (a) from the return belt to the inclined return belt

(b) from the inclined return belt to the vibrator (material spreader).

- Shadow sections between camera photo booth and robots-windows

.

- Other Requirements

- The offer will include equipment installation study and 3d draws.

- Delivery time: No later than 28.02.2023

- The Container with the integrated equipment should not exceed the permissing weight for its transport (28 tons)

- The delivery of the Container with the integrated equipment will take place in Heraklion, Crete.

- The delivery of the Inclined Feed Belt will take place in Kefalonia

- All metal parts will be sandblasted to Sa 2.5 (Swedish Standard) so that the difference between the depths and condition of the sandblasted surface does not exceed 75μm +-25%. The surfaces will be painted with a zinc-phosphorus or zinc-chromium epoxy paint base. This will be followed by two layers of final alkyd resins with the last layer being silicone. All materials should bear the appropriate markings, approvals, and carry the safety data sheet and the technical data sheet. Color RAL 5002

- The equipment should have an automatic lubrication system